MVP Prototype Plan

- Our intended solution is a lightweight, web-based platform that uses exported Building Automation System (BAS) trend data in CSV format to detect common HVAC faults early. The MVP should focus on explainable diagnostics, severity scoring, and clear “what to check next” recommendations. The goal is not to build a full enterprise product, but to prove that Fault Finder can take building data, detect important faults, rank them by urgency, and present the results in a simple dashboard and report that users can understand and act on.

- For the final pitch, the MVP must demonstrate the core workflow clearly. The system should allow a user to upload a CSV file, normalize and map the points, run rule-based fault detection, assign severity scores, and show the results on a clean dashboard. It should also generate a short report summarizing the major faults, likely causes, and recommended next actions. Based on the project work completed so far, key features already defined for the MVP include CSV ingestion, automated data normalization, fault detection, severity scoring, dashboard output, and performance summary reporting.

- The recommended MVP scope is as follows.

- First, Freeze the MVP workflow: Upload CSV → Map columns → Run fault checks → Show results → Export report

- Second, build a point-mapping step to handle different BAS naming conventions. (Not urgent)

- Third, implement a rule-based fault engine starting with 3–5 high-impact faults such as schedule mismatch (runtime outside occupied hours), simultaneous heating and cooling, short cycling (too many cycles/hour), sensor flatline (Temperature stuck), and high filter pressure drop. (We can do 3 or 4 only)

- Fourth, add severity scoring so users can see which issues are most urgent.

- Fifth, create plain-language recommendations that explain the fault and suggest what to check next.

- Sixth, complete a simple dashboard, using the current mockup as the visual target, showing active faults, trend charts, alerts, and equipment summary.

- Seventh, generate an automated weekly-style fault report for the pitch demo.